Representative Contact – PLPSoCal

Crissy Cosci – 626-482-8420

ccosci@plpsocal.com

Acorn Lamp Fixture

Description

Avenue Series B750-VCOB – Fixture shall have a textured acrylic acorn lens with a perforated brass decorative ring and a top finial with a Dark Bronze Smooth finish.

Fixture shall have the VCOB Led module with a 4000K color temperature and a minimum 70 CRI or better with a Symmetric and Asymmetric distribution options. LED shall offer a minimum life of 100,000 hours at L70. Fixture shall have a wattage of 56 and lumens per watt shall be 111 or better.

Fixture driver shall be high efficiency with a THD less than 20% and a high-power factor greater than .9 and be capable of a 0-10v signal.

Fixture manufacturer shall offer a seven-year limited warranty.

Acorn shall mount on a historic fluted and flared fitter. Fitter shall offer the TL (Twist-Lock) fitter shall have an aluminum die-cast twist-lock mechanism. The tool-less ¼ turn action allows for easy globe removal and replacement. A die-cast ring assembly is mechanically attached to the globe and is removable if the globe is broken or replaced.

Pole shall be 6200 Oxford Series and a standard height of 12ft. Pole shall be a cast tapered fluted pole with a one-piece decorative heavy wall 356 alloy cast aluminum base.

Fixture and pole shall be painted a Dark Bronze Smooth finish.

ORDER NUMBER:

SYMMETRIC OPTIC – (Light source distributes light evenly in all directions)

STERNBERG – PT-B750-5T-VCOB-4L-40-TS-MDL03-A-PBDR—GFF DB-DB-6212-TFP63-DB

ASYMMETRIC OPTIC – (Light source distributes light to one side)

STERNBERG – PT-B750-5T-VCOB-4L-40-TA-MDL03-A-PBDR—GFF DB-DB-6212-TFP63-DB